# Lithium Battery Fire Guidance for Fire Personnel

Adopted from the 2023 Massachusetts Advisory on the Dangers of Lithium Batteries

WARNING: Lithium-ion batteries can vent large amounts of flammable and toxic gases to create flammable and explosive atmospheres. Pressurized white “smoke” may vent moments prior to ignition of flammable/toxic gases.

# Vape Devices, Cell Phones, Tablets, Computers

- These types of devices vary in the number of lithium batteries they contain

- Use of improper charger can cause battery abuse

- Although battery packs are smaller in nature and contain fewer individual batteries than larger devices, the following precautions still apply

# Personal Mobility Devices/ E-Bikes

- Scooters, hover boards, bikes (may fold for storage/transport)

- Use of improper charger can cause battery abuse

- Battery Pack

  - Composed of multiple cells and a battery management system

  - May be located on floor board, frame rail, or rear rack of bike

  - Pack may be removable or installed

## Response Considerations

- Batteries on these devices can vent and/or catch fire from thermal, electrical, or physical abuse

- Abuse of lithium-ion battery cell can cause thermal runaway and propagate to other cells causing a slow “domino” effect

- Gases and vapors venting from the battery cells may be toxic and/or flammable

- Use CGL/LEL to check for flammable environments

- Avoid entering confined areas; do not take units on elevators

- Use appropriate PPE such as structural turnout gear and SCBA

- If charging, consider unplugging or shutting off power if no flammable environment exists

- Consider moving device from structure or away from exposures

## Fire Considerations

- Control fire with water, protect exposures, and cool battery cells

- Foam, CO2, Dry Chemical, and Class D extinguishers are ineffective

- Observe battery cells with Thermal Imaging Camera (TIC)

- Battery cells can ignite days after original incident

- Li-Ion batteries or mobility devices that are involved in fire, found within a fire area, or subject to elevated temperatures should be moved from the fire area as soon as possible. This should be done prior to overhaul when possible.

- A diligent search should be made for stray battery cells, which may have become dislodged during fire conditions and or extinguishment. These stray cells may become a source of ignition for a rekindled fire.

- Battery cells should be stored a safe distance from other combustibles.

# Electric Vehicles (EVs)

## Design/Construction

- 1500+ small lithium-ion battery cells or pouches are grouped together into battery modules

- Multiple battery modules are installed in the vehicle to make up the battery pack

- High-voltage cables are often colored-coded orange

## Response Considerations

- Use appropriate PPE such as structural turnout gear and SCBA

- Follow manufacturer (OEM) specific guidance

- Silent engine noise does not mean that the vehicle is off

- Stabilize the vehicle

  - Shift into park

  - Turn off vehicle

  - Chock all four tires

  - Remove key (fob, card, or owner’s phone)

- Batteries in EVs can vent or catch fire from thermal, electrical, or physical abuse

- Abuse of lithium-ion battery cells can cause thermal runaway and propagate to other cells

- Gases and vapors venting from battery cells may be toxic and/or flammable

- Use caution in confined areas including garages or shipping containers

- Use CGI/LEL to check flammable environments o If charging, consider unplugging or shutting off power if no flammable environment exists

- Check the area for battery cells that may have been ejected from the battery pack

- Ensure towing/recovery personnel are briefed on hazards including re-ignition or delayed ignition for both transport and storage

- Consider following the vehicle to the tow yard for vehicles involved in fire and or accidents with significant vehicle damage

- Vehicles should be visibly identified as a hybrid or electric vehicle (EV)

- Vehicles should be stored away from other vehicles, ideally 50 feet away from any combustibles. Larger EV’s may require further distance

- If an EV is towed and tarped outside to preserve evidence, be aware that gasses released from the battery may accumulate under the tarp

## Fire Considerations

- Select appropriate strategy (non-intervention, defensive, offensive)

- Consider controlling the fire with water, protecting exposures, and cooling battery cells versus controlled burn

- Observe battery cells with Thermal Imaging Camera (TIC)

- Consider waiting until no fire, smoke, audible hissing or heating present for at least 45 minutes before the vehicle is released

- Battery cells can vent/re-ignite days after original incident

- Foam, CO2, Dry Chemical, and Class D extinguishers are ineffective